Using a GPS to Collect Data on Vegetation Cover

Protocol

1. Make sure you have:

- GPS, Batteries

- Range Finder

- Compass

- Copies of the data sheets provided

2. Travel to the desired location and turn on the GPS.  Make sure the GPS sees at least 3 satellites.

3. Select one person to act as scribe and one to handle the GPS.  Rotate roles on each feature.

4. Go to the benchmark and record a way point.  Enter the number of the waypoint, easting, and northing, into the form.

5. Go to each of the other light poles and record a way point, easting, and northing.  Return to the benchmark between recording each location and take another waypoint.

6. Record the position of at least 3 other features that should be visible in the aerial photos with the GPS.

7. For using the range finder and compass, have each person use a different piece of equipment and then rotate the roles.

8. Return to your benchmark and shoot the location of the light poles and record the angle and distance to the light poles.

9. From your benchmark, shoot the location of the other features and record the angle and distance to the features.

10. Take a final waypoint at your benchmark.

Data Sheet

1. Date and time:

2. Data Collectors:

3. GPS Type:

4. Range Finder Type:

5. Compass Type:

6. Notes:

| GSP Measurements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target | Easting | Northing |  | Notes |
| Light Pole 1 |  |  |  |  |
| Light Pole 2 |  |  |  |  |
| Light Pole 3 |  |  |  |  |
| Light Pole 4 |  |  |  |  |
| Light Pole 5 |  |  |  |  |
| Light Pole 6 |  |  |  |  |
| Light Pole 7 |  |  |  |  |
| Light Pole 8 |  |  |  |  |
| Compass and Range Finder Measurements |  |  |  |  |
| Target | From | Angle | Distance |  |
| Light Pole 1 | Benchmark |  |  |  |
| Light Pole 2 | Benchmark |  |  |  |
| Light Pole 3 | Benchmark |  |  |  |
| Light Pole 4 | Benchmark |  |  |  |
| Light Pole 5 | Benchmark |  |  |  |
| Light Pole 6 | Benchmark |  |  |  |
| Light Pole 7 | Benchmark |  |  |  |
| Light Pole 8 | Benchmark |  |  |  |
| Benchmark Coordinates |  |  |  |  |
| Target | Easting | Northing |  | Notes |
| Benchmark |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark |  |  |  |  |